Q. Demonstrate the RIP Routing in Packet Tracer.

Aim: To configure and demonstrate RIP dynamic routing protocol between two networks using two routers in Packet Tracer.

Steps:

1.In packet tracer place 2 PCs and 2 Routers.

2.Connect Router0 with PC0 via FastEthernet0/0 using cross-copper cord and Router1 with PC1 via FastEthernet0/0 using cross-copper cord.

3.Connect both routers via FastEthernet0/1 using cross-copper cord.

4.Set the IP address and default gateway of both the PCs in IP configuration of both PCs respectively.

5.Configure RIP version 2 routing protocol on both routers.

Topology :

Code:

Router0 Configuration:

Router>enable

Router#configure terminal

Router(config)#hostname Router0

Router0(config)#interface FastEthernet0/0

Router0(config-if)#ip address 192.168.10.1 255.255.255.0

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#interface FastEthernet0/1

Router0(config-if)#ip address 10.10.10.1 255.255.255.0

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#router rip

Router0(config-router)#version 2

Router0(config-router)#network 192.168.10.0

Router0(config-router)#network 10.10.10.0

Router0(config-router)#no auto-summary

Router0(config-router)#exit

Router0(config)#exit

Router0#copy running-config startup-config

Router1 configuration:

Router>enable

Router#configure terminal

Router(config)#hostname Router1

Router1(config)#interface FastEthernet0/1

Router1(config-if)#ip address 10.10.10.2 255.255.255.0

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#interface FastEthernet0/0

Router1(config-if)#ip address 172.16.20.1 255.255.255.0

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#router rip

Router1(config-router)#version 2

Router1(config-router)#network 10.10.10.0

Router1(config-router)#network 172.16.20.0

Router1(config-router)#no auto-summary

Router1(config-router)#exit

Router1(config)#exit

Router1#copy running-config startup-config

PC0 Configuration:

IP Address: 192.168.10.2

Subnet Mask: 255.255.255.0 Default Gateway: 192.168.10.1 PC1 Configuration:

IP Address: 172.16.20.2

Subnet Mask: 255.255.255.0

Default Gateway: 172.16.20.1

PCO Command Prompt :

C:\>ipconfig

C:\>ping 192.168.10.1

C:\>ping 10.10.10.1

C:\>ping 10.10.10.2

C:\>ping 172.16.20.1

C:\>ping 172.16.20.2

PCO Command Prompt :

C:\>ipconfig

C:\>ping 172.16.20.1

C:\>ping 10.10.10.2

C:\>ping 10.10.10.1

C:\>ping 192.168.10.1

C:\>ping 192.168.10.2

Router Verification Commands:

Router0#show ip route

Router0#show ip protocols

Router0#show ip rip database

Router1#show ip route Router1#show ip protocols

OUTPUT :

PC0 OUTPUT :

PC1 OUTPUT :

IP ROUTE OUTPUT :

RIP PROTOCOL OUTPUT :

CONCLUSION : The above network topology has been executed successfully and RIP version 2 dynamic routing has been demonstrated.