Q. Demonstrate Telnet in Packet Tracer.

Aim: To configure Telnet on routers to enable remote administration and test remote connectivity between devices.

Devices Needed:

2 Routers

- Switch

- PCs (Telnet Clients) Connection Guide:

Connect Router0 and Router1 via serial cable (Serial DCE)

Connect Router0 to Switch via FastEthernet

Connect PCs to Switch via copper straight-through cables Topology:

Code:

Router1 Configuration:

Router>enable

Router#configure terminal

Router(config)#hostname Router0

Router0(config)#interface FastEthernet0/0

Router0(config-if)#ip address 192.168.1.1 255.255.255.0

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#interface Serial0/0/0

Router0(config-if)#ip address 10.0.0.1 255.255.255.252

Router0(config-if)#clock rate 64000

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#line vty 0 4

Router0(config-line)#password cisco

Router0(config-line)#login

Router0(config-line)#exit

Router0(config)#enable secret class

Router0(config)#ip route 0.0.0.0 0.0.0.0 10.0.0.2

Router0(config)#exit

Router0#copy running-config startup-config

Router2 Configuration:

Router>enable

Router#configure terminal

Router(config)#hostname Router1

Router1(config)#interface Serial0/0/0

Router1(config-if)#ip address 10.0.0.2 255.255.255.252

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#interface FastEthernet0/0

Router1(config-if)#ip address 172.16.1.1 255.255.255.0

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#line vty 0 4

Router1(config-line)#password cisco

Router1(config-line)#login

Router1(config-line)#exit

Router1(config)#enable secret class

Router1(config)#ip route 192.168.1.0 255.255.255.0 10.0.0.1

Router1(config)#exit

Router1#copy running-config startup-config PC1 Configuration:

IP Address: 192.168.1.10

Subnet Mask: 255.255.255.0

# Default Gateway: 192.168.1.1

PC2 Configuration:

IP Address: 192.168.1.11

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

Verification: In PC Command Prompt PC1 Telnet Tests:

C:\>ping 192.168.1.1

C:\>ping 10.0.0.2

C:\>ping 172.16.1.1

C:\>telnet 192.168.1.1

C:\>telnet 10.0.0.2

PC2 Telnet Tests:

C:\>ping 192.168.1.1

C:\>ping 10.0.0.2

# C:\>ping 172.16.1.1

# C:\>telnet 192.168.1.1

C:\>telnet 172.16.1.1

Router Verification Commands:

Router#show users

Router#show line vty 0

Router#show running-config | include vty Output:

PC1 Telnet to Router1: telnet 192.168.1.1

PC1 Telnet to Router2:

# C:\>telnet 10.0.0.2

PC2 Telnet to Router1:

C:\>telnet 192.168.1.1

Ping Test Results:

Router Show Users Output:

Conclusion: Telnet has been configured successfully on both routers.